Water Quality Excursion and Exceedance Analysis based on Magnitude, Duration, and Frequency: Batch Analysis

The file downloaded from the Batch Analysis tab is a ZIP file containing results of the water quality excursion and exceedance analysis based on magnitude, duration, and frequency from the criteria of a state/tribe.

- List of Files
1. TADAShinyAnalyze_batch_analysis_result.csv

The dataset with the duration analysis result.

2. TADAShinyAnalyze_batch_analysis_summary.csv

The summary dataset with the frequency analysis result.

3. TADAShinyAnalyze_prog.rda

The R Data file stores the datasets and session information.

- Analysis Steps

- Step 1: Filters criteria table for specific state/tribe, which contains magnitude, duration, and frequency. Joins the criteria table to

Step 2: Joins pH, temperature, and hardness measurements to main dataset as covariates. Uses plus/minus one day temporal window to match nearby measurements. These covariates are essential for equation-based criteria calculations

Step 3: Data Integration

•	Links monitoring locations to assessment units

•	Joins designated uses to each assessment unit

•	Matches applicable water quality criteria to each data point

•	Results in expanded dataset with all relevant regulatory context

Exceedance Calculation Phase (Steps 4-5)

Step 4: Non-Equation Criteria Evaluation

•	Processes fixed threshold criteria (simple numeric limits)

•	Compares measured values against upper/lower bounds

•	Flags excursions when values fall outside acceptable range

Step 5: Equation-Based Criteria Calculation

•	Handles dynamic criteria that depend on water chemistry:

o	Hardness-dependent: Common for metals criteria

o	pH-dependent: For parameters affected by acidity

o	pH + Hardness combined: For complex interactions

o	pH + Temperature: For temperature-sensitive parameters

•	Calculates site-specific thresholds using measured covariates

•	Evaluates excursions against calculated thresholds

Aggregation Phase (Steps 6-7)

Step 6: Initial Summary Statistics

•	Aggregates excursion data by selected analysis unit (Site/AU/Criteria Group)

•	Calculates sample counts, date ranges, and basic statistics

•	Determines excursion frequency and percentage

Step 7: Time Series Standardization

•	Collapses duplicate samples at same timestamp (averaging)

•	Creates canonical time series:

o	Hourly series: For sub-daily criteria

o	Daily series: For daily/monthly/seasonal criteria

•	Prepares data for rolling window calculations

Duration Analysis Phase (Steps 8-9)

Step 8: Duration Window Calculation

•	Applies rolling windows based on duration requirements (e.g., 30-day average)

•	Calculates specified statistics within each window:

o	Arithmetic mean/min/max

o	Geometric mean

o	Percentiles

•	Updates equation-based thresholds using windowed covariate values

Step 9: Frequency Assessment

•	Evaluates compliance with frequency-based criteria:

o	Number not meeting: Simple exceedance count

o	Percent not meeting: Exceedance percentage threshold

o	N-samples in 3 years: Rolling 3-year maximum

o	Percentile method: Statistical distribution approach

•	Determines final compliance status (Exceed/Not Exceed)

Results Compilation Phase (Steps 10-11)

Step 10: Results Integration

•	Joins duration and frequency assessments

•	Combines with original excursion summary

•	Creates comprehensive compliance dataset

Step 11: Output Simplification

•	Formats duration/frequency information into readable strings

•	Removes intermediate calculation columns

•	Produces final user-friendly assessment results

Key Analysis Features

The pipeline handles three regulatory complexity levels:

1.	Magnitude: Direct comparison to thresholds

2.	Duration: Time-averaged compliance (e.g., "30-day mean not to exceed X")

3.	Frequency: Allowable exceedance rates (e.g., "not more than once in 3 years")

The modular design allows for:

•	Multiple spatial aggregation levels

•	Various statistical methods

•	Dynamic criteria based on site conditions

•	Complex regulatory logic implementation