Water Quality Excursion and Exceedance Analysis based on Magnitude, Duration, and Frequency: Custom Analysis

The file downloaded from the Custom Analysis tab is a ZIP file containing results of the water quality excursion and exceedance analysis based on magnitude, duration, and frequency from the criteria of a state/tribe.

List of Files
1. TADAShinyAnalyze_custom_analysis_result.csv

The dataset with the duration analysis result.

2. TADAShinyAnalyze_custom_analysis_summary.csv

The summary dataset with the frequency analysis result.

3. TADAShinyAnalyze_prog.rda

The R Data file stores the datasets and session information.

Analysis Steps

  - Filter the criteria table for the selected state/tribe. The table includes information on magnitude, duration, and frequency.

  - Join pH, temperature, and hardness measurements to the main dataset as covariates, which are essential for equation-based criteria calculations, if they are available. A ±1-day temporal window is applied to match nearby measurements.

  - Determines the spatial analysis unit as Monitoring location ID, Assessment Unit, or Custom Grouping.

  - Join the criteria table to the water quality dataset using the parameter name (TADA.CharacteristicName) and unit (TADA.ResultMeasure.MeasureUnitCode in the water quality dataset and MagnitudeUnit in the criteria table) as the primary join keys. Optionally, assessment use (ATTAINS.UseName) and fraction (TADA.ResultSampleFractionText) can be included as additional join keys.

  - Calculate the excursion of each water quality measurement record. For non-equation-based criteria, compare the measured value to the magnitude threshold. For equation-based criteria, calculate the threshold for the following equation types: Hardness-dependent, pH-dependent, pH and Hardness dependent, and pH and Temperature dependent. then compare the measured value to the threshold. Flag excursions when values fall outside acceptable range.

  - Summarize the excursion dataset. Aggregate excursion data by the selected analysis unit. Calculate the sample counts, date ranges, and basic statistics. Determine excursion frequency and percentage.

  - Aggregate the time series. Collapse duplicate samples at the same dates and times. For sub-daily criteria, calculate the hourly time series of measured values. For daily/monthly/seasonal criteria, calculate the daily series of measure values. Aggregate the time series based on the spatial unit (Monitoring location ID, Assessment Unit, or Custom Grouping). Conducted the same aggregation for available covariates.

  - Apply the duration calculation based on the duration information. The DurationValue and DurationUnit determines the window requirements (e.g., 1-day, 7-day, 30-day, …). The DurationMethod specfies the summary statistics the following summary statistics with the specific rolling window requirement.

    - Arithmetic mean/min/max:

    - Rolling arithmetic mean (Similar to arithmetic mean)

    - Geometric mean

    - Arithmetic extremes

  - Update the magnitude threshold based on the new daily and monthly dataset. For non-equation-based criteria, use the original magnitude threshold. For equation-based criteria, calculate the threshold based on the available covariates.

  - Conduct the frequency assessment based on the frequency information (FreqValue and FreqMethod). The FreqMethod specifies the following frequency assessment methods, while FreqValue documents the associated key values to use.

    - Number not meeting: Use a simple excursion count to determine exceedance.

    - Percent of samples not meeting: Calculate the percentage of samples not meeting the criteria to determine exceedance.

    - Percentile method: Calculate the percentile of the sample and compare with the magnitude threshold.

  - Summarize the frequency analysis and determine the Exceed/Not Exceed status of each spatial unit.

  - Combine the duration and frequency assessment summary with the original excursion summary.